Renault 5 Home-Assistant Custom Dashboard Installation

Version: V03-0126

Where possible, I have tried to make these instructions as clear as I can. You may need to modify the location of existing files or folders in your instance of Home Assistant, in order to ensure this card’s functionality.

DISCLAIMER

WARNING! This ZIP file and these instructions are utilized “at your own risk” and if, as a consequence of implementing any changes, it renders your instance of HA un-useable, I will NOT be held responsible. As a sensible precaution, please ensure you perform a full backup of your HA instance BEFORE undertaking these changes. Note: at present the Start/Stop Charging functions are not yet functional, due to the current limitations of the Renault API.

About This Custom Dashboard

This dashboard is designed to work without the use of the official Home Assistant Renault integration. Instead, it uses the renault-api CLI inside a Python ‘venv’ (virtual environment) and a set of shell scripts that:

- fetch RAW data from Renault endpoints

- never hang Home Assistant (timeouts)

- never block waiting for interactive prompts (</dev/null + prompt detection)

- fall back to cached JSON data cleanly, when the API cannot be accessed

- publish consistent JSON attributes for template sensors and MQTT

What You Get In The ZIP (Main Components)

- Shell scripts (in the format get_r5_*.sh) that call the Renault API

- Conventional scripts that run specific tasks

- Shell commands that gather the API data and feed front end sensors

- Command-line actions (HVAC start/stop, horn/lights, charging start/stop) that are safe in non-interactive HA context

- Template sensors that decode the RAW JSON into human-friendly entities to display on your dashboard

- Images to display on your front end for your specific car as well as colour matching map markers and a background image

- Optional MQTT publishing (for the Google Map card to track your car's position)

- CSS and font files for the front end cosmetics

- Automations to run specific tasks such as capturing the start and end of a charge session and displaying the data on your dashboard

Pre-Requisites

This guide assumes that you have pre-installed the following features and add-ons into your HA instance. All of these are necessary for the card to function correctly. It is further assumed that you also have a working knowledge of HA, in order to install these add-ons and features. It is beyond the scope of this guide  to show you how to install them. Safe to say there is plenty of guidance available online, should you need it. I would strongly advise you to install them before proceeding further.

- HACS

- File Editor (Add-on)

- Google Map Card by KATA7 (via HACS)

- Mushroom Cards (via HACS)

- Button Cards (via HACS)

- Your own EV Home Charger’s HA integration

- Advanced SSH & Web Terminal (Add-on)

- Mosquitto MQTT broker (Add-on)

Installing the Renault API

Note: In order to install the API, you must first have installed the add-on "Advanced SSH & Web Terminal" into your instance of Home Assistant. Without it, you will NOT be able to proceed any further. Once installed you can proceed to step 1).

What follows are a set of instructions on how to install the API. These may vary depending on how you have set up your instance of HA, for example if you are running it in ‘Docker’ or on a VM. In any case, the aim is to install the API in a “virtual environment” in HA (signified when you see ‘venv:’ at the command prompt within the ‘Advanced SSH & Web Terminal’ add-on, when you are done) Once successfully installed, you can interact with the Renault CLI and retrieve information from your car, in much the same way that the “My Renault” and “Kelec” mobile apps do.

Step (1)

From the Advanced ‘SSH & Web Terminal’ add-on in HA, the first step is to create a "venv" (virtual environment) folder. Copy and paste all these commands, as one block, into the terminal then press enter.

mkdir -p /config/renault

python3 -m venv /config/renault/.venv

. /config/renault/.venv/bin/activate

Once you have done that, you should see the prompt change to "venv" or "venv renault". If so, we can now proceed to installing the Renault API, by copying and pasting these commands, as one block into the terminal, then pressing enter.

python3 -m pip install --upgrade pip wheel

python3 -m pip install "renault-api[cli]"

renault-api –version

Note that this last line of code has 2 dashes before “version”, which may not be immediately apparent. Once the API has been installed, you will need to create a credentials file. This is used by the scripts to validate your identity with Renault and allows you access to your car's specific data.

Step (2)

First time login (creates credentials file). All scripts force HOME=/config/renault so the CLI stores credentials here: /config/renault/.credentials/renault-api.json

Run the interactive login once, by copying and pasting these commands as one block into the terminal, then press enter.

export HOME="/config/renault"

export XDG_CONFIG_HOME="/config/renault"

export XDG_DATA_HOME="/config/renault"

. /config/renault/.venv/bin/activate

renault-api --log status

You’ll be prompted for:

- locale (e.g. ‘en_GB’ if you are in the UK)

- username + password (these are the same as those you use to login to the 'My Renault' app)

- account selection (if multiple)

- vehicle selection (if multiple)

Note: if you are not UK based, there is a list of all the locales available towards the end of this document under “Customizations”

Step (3)

After that, confirm the credentials file exists using this command (copy and paste it into the terminal and press enter.

ls -la /config/renault/.credentials/

Once your credentials file has been created, you are ready to move on to the installation of the ZIP files. Just before you do, you must extract the Kamereon account id that the API created for you. You will need this later on, when overwriting some redacted code. From the ‘venv’ prompt, copy the following command and paste it into the terminal, then press enter:

sed -n 's/.*\("kamereon_account_id":"[^"]*"\).*/\1/p' /config/.storage/core.config_entries | head -n 1

This will extract your “Kamereon account id”. Make a note of the id/number by copying and pasting it into a plain text file on your laptop or PC.

Installing The ZIP files

Before proceeding further, you must have already installed the add-on in HA called "File Editor”. Once installed, proceed as follows.

Step (1)

Ensure you have extracted the contents of the ZIP file into a suitable location on your laptop or PC. Once done, you will see it is comprised of several folders.

- Automations

- CSS

- Fonts

- Helpers

- Images

- MQTT

- Packages

- Scripts

- Templates

- YAML

I will go through each folder one by one and show you how to install their contents into HA. First, I need to explain one key point.

** Important Note: Several files in this ZIP contain redacted code (that is to say my personal details have been removed and replaced with a placeholder).

Where this is the case, you will need to replace the redacted code with your own details. Failure to do so will render the card inoperative. Redacted code will be shown in this format, for example:

<your-vin-number>

You need to replace this with your own details (you DO NOT need to include the chevrons, when inserting your details). For future reference when the instructions mention a file containing redacted code, I will annotate a reference to this note by denoting it thus: *REDCT*, to remind you that you need to edit the file accordingly, before you install it in Home Assistant

Step (2)

On your laptop or PC, navigate to the “Automations” folder you extracted from the ZIP file. In it you will see there are 16 files in total, numbered from ‘00’ to ‘15’. Note: I would strongly advise you to open all files with a plain text editor such as Windows “Notepad” or the Apple equivalent application.

Do NOT use MS Word or an equivalent word processing application, else the format of these files can become corrupted. This applies to all the contents of this ZIP file (except any images) not just the automation files.

Step (3)

Open the file automation (00) *REDCT*

From your text editor you will first need to modify any redacted code (in this instance you will see somewhere in the code “<your-mobile-id>”.  You will need to replace it with the entity id HA assigns your mobile (if you are using HA’s “Companion” app). You can find this by going to the “Settings” menu in Home Assistant, then “Devices & Services”, then look for the “Mobile App” integration. Under “Devices” you should see your mobile listed. Click on it and then pick any entity listed for your phone and click on it. A pop-up box will appear. Click on the gear icon to reveal that entity’s ID. The device ID is the portion that will look something like “sm_s938b” (if you are for example using a Samsung phone). Copy and paste only this element over the top of the redacted code (including the chevrons)

Step (4)

Once done, highlight the entire contents of the automation (00) file, ensuring you select ALL the code and then copy it.

Step (5)

Now in Home Assistant go to “Automations & Scenes”. Once there, in the bottom-right hand corner, click on the blue “Create automation” button, then in the pop-up, select “create new automation”. Now click on the three dots at the top right of the screen and select “Edit in YAML”

Once selected you will see a few lines of code appear, similar to this:

description: ""

mode: single

triggers: []

conditions: []

actions: []

Don’t worry about these right now, as we are going to paste over them.

Now highlight these five lines of code and paste the contents of your automation (00) you just copied from your text editor, over the top of them. Then you will see a “Save” button appear. Press it. It may ask you to change the name of the file to something else, just ignore the message and click OK or Save. Congratulations! You have just created the first automation which should appear as:

“(0) R5 - RAW Sensors Unavailable”

If for any reason HA will not let you save the copied text as an automation, it is highly likely you have not copied the entire code across from the text editor OR you have inadvertently changed or corrupted the format/layout/indentation in some way. Or, it might be that you simply clipped some of the code, whilst copying it across.

Home Assistant is VERY picky and will reject a file if it does not comply EXACTLY with the correct spacing/indentation and layout it wants. If you are still struggling, you will have to re-extract a fresh copy of the automation file from the ZIP file, onto your laptop or PC and try again, remembering to replace the redacted code, without disturbing the code around it.

Now repeat this process for each of the remaining automation files. Remember to open each in turn, check for any redacted code if present, modify it, being careful not to change any of the surrounding code, then highlight and copy the entire contents of each file and paste it into HA as a new automation, just as described for automation (00)

Note that redacted code within this ZIP’s files, can take the form of

<your-mobile-id>		  replace with ID HA uses for your mobile (see earlier)

<your-VIN-number>		  replace with your own car's VIN number

<your-account-number>	  replace with your Kamereon account id

<your-username>		  replace with the username you use for the My Renault app

<your-password>		  replace with the password you use for the My Renault app

<your-google-maps-api-key>    replace with your own Google Maps API key

This last one (see below) should be changed to the entity ID used by whichever EV home charger integration you are currently using in Home Assistant, that represents its power output (kw) WHEN CHARGING. Currently there are several HA integrations for most of the popular chargers such as Hypervolt, Ohme, Peblar, Wallbox, Zaptec etc

<ev-charger-power-output>

Change this to for example: hypervolt_ev_power or whatever the entity is called in your EV charger’s HA integration, that represents power output when charging.

Just be sure to check each automation file and change it accordingly. As mentioned if a file contains redacted code it will be signified by *REDCT* in this guide. For ease of use however, I have indicated which of the automation files contain *REDCT* code and how many instances each file contains.

Automation files:

(00) 1 instance, <your-mobile-id>

(01) 3 instances, <ev-charger-power>

(02) 1 instance, <ev-charger-power>

(03) 1 instance, <ev-charger-power>

(04) 3 instances, <ev-charger-power>

(05) 1 instance, <ev-charger-power>

(08) 2 instances, <your-account-number> + <your-vin-number>

(10) 1 instance, <your-mobile-id>

(11) 2 instances, <ev-charger-power>

(12) 2 instances, <ev-charger-power>

(15) 1 instance, <ev-charger-power>

Step (6)

Open the “File Editor” add on in Home Assistant

If you are not familiar with using the File Editor in Home Assistant, above the list of folders and files that make up your Home Assistance installation, you will see four buttons. From left to right “New File”, “New Folder”, “Upload File”, “GIT”.

Step (7)

Ensure you are currently in the root (often known as “config”) directory and click on the New Folder button and type in “www”

Step (8)

Staying within File Editor, open the “www” folder you just created and then click on the Upload File button. Navigate to the CSS folder in this ZIP file and upload the file zen-dots.css directly into the new ‘www’ folder.

Step (9)

Staying in the www directory, create two new folders: “backgrounds” and “fonts”. Next open the fonts directory and click on Upload File button. Navigate to the “Fonts” directory in this ZIP file. Select the file ZenDots-Regular.ttf and press OK to upload it.

Step (10)

Next return to the root directory and create these five folders. “helpers”, “mqtt”, “packages”, “scripts” and finally “templates”.

Step (11)

Go to the ‘helpers’ folder you just created. And click on the upload file button. Navigate to the “Helpers” folder in the zip file and one by one upload all the following files:

input_boolean.yaml

input_datetime.yaml

input_number.yaml

input_text.yaml

Step (12)

Go to the “www/backgrounds” folder your created earlier and click on the Upload File button. Navigate to the “Images” folder in the ZIP file, then select the subfolder “Background” and in that folder select the file card-background.jpg. Upload it.

Step (13)

Staying in the “www/backgrounds” folder, click on the Upload File button again. Navigate to the “Images” folder in the ZIP file, then select the subfolder “Map Markers” and in that folder select the file that MATCHES the colour of your car (there are six choices) ie: map-marker-pop-yellow.png. Upload the correct one. Next, navigate to the “Images” folder in the ZIP file, then select the subfolder “Charging” and upload the image charge-indicator.png within it.

Step (14)

Staying in the “www/backgrounds” folder, click on the Upload File button again. Navigate to the “Images” folder in the ZIP file, then select the sub-folder “Models”. Within that folder you will see four further sub-folders: “Evolution”, “Iconic”, “Roland Garros” and “Techno”. Open the folder that matches your car and then pick the one image to upload, that matches your exact trim level. In total there are 22 available options across the four variants, so choose carefully! Once you find the right one, upload it.

Step (15)

Return to the root directory in Home Assistant and navigate to the “mqtt” folder you created. Click on upload file button and navigate to the “MQTT” folder in this ZIP file and upload the renault-5.yaml file.

Step (16)

Return to the root directory in Home Assistant and navigate to the “packages” folder you created. In it click the Upload File button and navigate to the “Packages” folder in this ZIP file. Upload the file r5_test.yaml

Step (17)

Open your text editor on your laptop or PC. Navigate to the “Scripts” folder in this ZIP file. Within that folder open: “renault5-scripts.yaml” *REDCT*

It has 13 instances, you need to modify. (12 instances of <your-mobile-id> and 1 instance of <ev- charger-power> You can, if desired, use “find/replace” to quickly search for each instance and replace them with your details as explained earlier in the guide. Once modified, save the file, return to the root directory in Home Assistant and navigate to the “scripts” folder you created. In it click the Upload File button then navigate back to the “Scripts” folder in this ZIP file. Within that folder select: the file you just modified “renault5-scripts.yaml” and upload it.

Step (18)

Whilst in the “scripts” folder in HA (from the “File Editor”) you need to next upload all the shell script files. These files are crucial, as these are the ones that retrieve your car’s data from the Renault API. A word of caution here. Windows can sometimes add hidden characters to these files (known as CRLF) if you open them in a text editor. *** Later in the guide I will show you how to strip these out if they do not work as intended, by running some commands from the “Advanced SSH & Web Terminal”. For now, you will need to initially replace the redacted code in each of them. In almost every case this entails overwriting the redacted code with your VIN number and/or your account number where indicated. Check each .sh file in turn and overwrite the redacted code as explained earlier. Save each file as you go, being cautious not to disturb the layout/indentation of the file structure. Remember you only need to replace the chevrons and everything inside them. Do NOT remove any other code, so for instance if the redacted code is inside quotation marks or single quotation marks, these MUST remain in place. Ie “<your-vin number>” becomes “VSP12345678”. As I said these files are the brains behind the whole card, so take your time. Note: any VIN number letters MUST be uppercase.

Of the sh files, these are the redacted instances you need to overwrite

Battery: 3 instances - <your-account-number> and 2 x <your-vin-number>

Charges: 3 instances - <your-account-number> and 2 x <your-vin-number>

Cockpit: 3 instances - <your-account-number> and 2 x <your-vin-number>

EV Settings: 3 instances - <your-account-number> and 2 x <your-vin-number>

HVAC: 3 instances - <your-account-number> and 2 x <your-vin-number>

Location: 3 instances - <your-account-number> and 2 x <your-vin-number>

SOC Levels: 3 instances - <your-account-number> and 2 x <your-vin-number>

API LOGIN: 2 instances - <your-username> and <your-password>

Once you have changed the redacted code and saved each file and whilst in the “scripts” directory, you can then upload each file as per previous instructions.

Step (19)

*** There are a couple more things to do in the “Advanced SSH & Web Terminal” that you must now complete. You need to ensure that while you edited the .sh files you did not inadvertently introduce any hidden CLRF characters into the code. (sometimes happens when you edit a file in a text editor in Windows)

From the “venv” prompt copy and paste in this following command and press enter. This will strip out these characters:

sed -i 's/\r$//' /config/scripts/get_r5_*.sh

Then repeat it for the login.sh file

sed -i 's/\r$//' /config/scripts/r5_api_login.sh

Next, make all the .sh files executable, by typing in this command and pressing enter:

chmod +x /config/scripts/get_r5_*.sh

Repeat this for the login.sh file

chmod +x /config/scripts/r5_api_login.sh

It is recommended that you now restart Home Assistant before you continue with the installation, as this will refresh the script files and reload them.

Step (20)

Next up navigate to the “Templates” folder in this ZIP file. In it you will see two files:

“r5_template_sensors.yaml” *REDCT*

“r5_test_panel_timers.yaml”

Open “r5_template_sensors.yaml” and replace any redacted code as explained earlier. Note that some of the sensors in this file, refer to LHD cars or cars that are equipped with a 40KwH battery or display KM as opposed to miles. I will come on to this later.

This file contains the following redacted code:

3 instances: <ev-charger-power>

Once you have done this, just as before, from “File Editor” navigate to the “templates” folder you created in Home Assistant. Click on the “Upload File” button and then navigate to the folder “Templates” in this ZIP file. Select each file in turn and upload them.

Step (21)

Now we can move on to the final folder in the ZIP file, “YAML”

In this folder you will find 5 files:

“command_line.yaml”

“config-entries.yaml”

“secrets.yaml” *REDCT*

“shell_command.yaml”

“front-end.text” *REDCT*

Within “File Editor” navigate to the root (config) directory in Home Assistant. From there click on the Upload File button and navigate to the “YAML” folder in this ZIP file. Select “command_line.yaml” and upload it. Repeat this process for “shell_command.yaml”. Next, within your text editor, open the file “secrets.yaml” from this same ZIP folder

You will notice that it contains these 4 lines containing redacted code.

renault_account_id: <your-account-number>

renault_vin: <your-vin-number>

renault_username: <your-username>

renault_password: <your-password> J

Just as before, you will need to overwrite all four instances with your own data.

Note: Your “renault_account_id” is actually your Kamereon account id that you extracted earlier in the guide, from the SSH Terminal. Just overwrite the <your-account-number> element with the details you saved to your laptop or PC.  Your vin number will contain some letters as well as numbers. Any letters must be ALL uppercase. Your username and password are the same ones you use to login to the My Renault app. Once you have saved the file, after overwriting the redacted code, you can upload it to the root directory, as per the previous two files. Next from your text editor, open the ZIP file “config-entries.yaml” in the “YAML” folder.

In it you will see the following lines of code, which comprises of 3 sections.

# CSS Font Styling

lovelace:

resources:

- url: /local/zen-dots.css

type: css

# Map Marker & Charge Testing Package

homeassistant:

customize:

device_tracker.r5_car_location_raw:

entity_picture: /local/backgrounds/map-marker-pop-yellow.png

packages: !include_dir_named packages

# Additional YAML Files

script: !include_dir_merge_named scripts

template: !include_dir_merge_list templates

mqtt: !include_dir_merge_list mqtt

shell_command: !include shell_command.yaml

command_line: !include command_line.yaml

input_datetime: !include helpers/input_datetime.yaml

input_number:   !include helpers/input_number.yaml

input_text:     !include helpers/input_text.yaml

input_boolean:  !include helpers/input_boolean.yaml

Now from “File Editor”, navigate to your root directory in Home Assistant and open the file “configuration.yaml”

Check to see if similar entries/headings already exist in that file already, such as ‘lovelace: resources:’ or ‘homeassistant: customize:’

If not, copy and paste the whole block of code above into the configuration.yaml file and save it

If any are already present, you will need to adjust your existing code accordingly to avoid duplication and allowing you to incorporate these new entries.

A note regarding this line of code:

entity_picture: /local/backgrounds/map-marker-pop-yellow.png

Note: you will need to change the file name to that of the map-marker image you uploaded earlier in this guide.

For example change:

map-marker-pop-yellow.png

to this

map-marker-white.png

In summary, when pasting this code into your configuration.yaml file in Home Asssitant, you just need to ensure you do not duplicate any code already there and always respect the indentation rules, exactly as written in the blocks above. Else you will not be allowed to save the file. DO NOT modify any other code in your configuration.yaml file, else doing so could cause your instance of HA to crash or prevent it from starting.

Step (22)

All that is left to copy now is the “front-end.text” file. We will come onto that shortly. For now, we can turn our attention to the front end (UI) of Home Assistant.

Creating The Front End Card

Note: The front end card has been styled to work best in Home Assistant  when you are running the default dark theme. If you are not you may wish to change some of the card mod styling to suit your own purposes.

Step (1)

From the “Overview” tab or wherever your main dashboard views are displayed, we are going to create a new view. Click on the PEN icon in the top right corner of the screen (“Edit Dashboard”)

Step (2)

Click on the PLUS icon at the top right of the screen (“Add View”). A pop-up dialog box will appear.

Step (3)

Under the “Settings” tab, the layout type should be set to ‘Sections (default). Type in a title for this new card such as “Renault 5” (the URL path will populate with the same name as you do). Lastly set the number of columns to ‘3’.

Step (4)

Under the “Background” tab you will see a box where it says ‘add picture’. Click where it says “select from media” and then click the folder “Image Upload”. You should see the three image files you uploaded earlier in this guide. Just select “card-background.jpg”.

Step (5)

Before saving this new dashboard view, set the opacity of the image to “10” and the image to “Fixed”. Leave the remaining options as is

Step (6)

Now save this view. Finally click the “Done” button, to exit editing mode in HA. You should now see your new front end card with the Renault 5 background but nothing else on it. We will sort that out shortly.

Step (7)

Using your text editor, open the file “front-end.text”  in the “YAML” folder in this ZIP. Now search for this redacted code and replace it with your own GoogleMaps API key

<your-google-maps-api-key>

(instructions on how to obtain a Google Maps API key are available online)

Step (8)

Search for this line of code and change the image file name to the one you uploaded earlier in this guide (that matches your car’s trim level)

image: /local/backgrounds/pop-yellow-iconic.png

so this might become:

image: /local/backgrounds/midnight-blue-roland-garros.png

Step (9)

Once you have personalized the code save the file.

Step (10)

Once more, from the “Overview” tab or wherever your main dashboard views are displayed, click on the PEN icon in the top right corner of the screen (“Edit Dashboard”) Click on the tab (view) called “Renault 5” you created and then the orange PEN icon next to it.

Step (11)

The pop up dialog box you saw earlier will appear. Now click on the three dots at the top right of the dialog box and select “Edit in YAML”. In that box you will see a section of code that looks something like this:

background:

opacity: 10

alignment: center

size: cover

repeat: repeat

attachment: scroll

image:

media_content_id: media-source://image_upload/8376522bc4e30b62fc5ecbbb3c20e168

media_content_type: image/jpeg

metadata:

title: renault5-background.jpg

thumbnail: /api/image/serve/8376522bc4e30b62fc5ecbbb3c20e168/256x256

media_class: image

navigateIds:

- {}

- media_content_type: app

media_content_id: media-source://image_upload

cards: []

title: Renault 5

Copy this entire block and save it in a text editor temporarily as a new file.

Step (12)

Open the “front-end.txt” file, highlight all the code and copy it.

Step (13)

In the pop up dialog box delete the code and then paste the card-yaml.text in it’s place.

Now scroll down to the bottom of the code you just pasted into the box until you reach these lines:

ha-state-icon {

{% if states('binary_sensor.r5_renault_raw_stale') =='on' %}

color: red;

animation: pulse 1s linear infinite;

{% else %}

color: grey;

opacity: 0.4;

animation: none

{% endif %}

}

ha-card {

background: none !important;

box-shadow: none !important;

border: none !important;

}

@keyframes pulse {

0% { transform: scale(1); opacity: 1; }

50% { transform: scale(1.08); opacity: 0.85; }

100% { transform: scale(1); opacity: 1; }

}

Step (13)

Immediately under the very last line of this code (namely the “}” symbol), paste in the snippet of code you copied in Step 11

Step (14)

Press the Save button, to close the dialog box and then the “Done” button at the top right of the screen. Your card should now be complete!

Customizations

This card is set up for a Renault 5 UK based right hand drive car with a 52kWh battery. If you are not UK based, and/or your car is left hand drive or it is equipped with a 40kWh battery, or you wish the odometer to display in km instead of miles, there are some specific changes to the code you need to now make.

Non GB based car

If you car is not UK based, you will need to make some changes to the following files.

Step (1)

From “File Editor” navigate to the “scripts” directory. Open the file “get_r5_battery.sh”. Click on the search icon in the file content panel at the top right of the screen and in it type “GB”. You will need to replace “GB” with the your correct country code (your “locale” as Renault term it!). Be careful not to disturb or move any text around it. JUST replace the two letters, that is all. These are the locales that Renault currently accepts.

bg_BG, cs_CZ, da_DK, de_AT, de_CH, de_DE, en_GB, en_IE, es_ES, es_MX

fi_FI, fr_BE, fr_CH, fr_FR, fr_LU, hr_HR, hu_HU, it_CH, it_IT, nl_BE, nl_NL

no_NO, pl_PL, pt_PT, ro_RO, ru_RU, sk_SK, sl_SI, sv_SE

Note that you only need to use the last two letters in this instance. So for example if you are based in Finland (locale code: fi_FI), type in ‘FI’ in place of ‘GB’. Once you have done this save the file. Repeat this process only for the following .sh files: Charges, Cockpit, HVAC, and Location. Once again, ensure that you only change the two letters “GB” in the code and NOTHING else.

Step (2)

From “File Editor” navigate to the “root” directory. Open the file “shell_command.yaml”.

Just as before, you will see there are instances of “GB” in the code. You need to replace each one just as you did in Step (1) with your locale code for your country. An easy way to do this is to use the “Search” icon at the top right of the screen. Click on it and type in “GB”. You will be given the option to replace one instance at a time or all 13. In the “Replace with” box type in your country code (ie “FI”) then click on “All”. Once you have done that, save the file and exit.

Left Hand Drive Car

If your car is left hand drive there are a couple of small changes you need to make to the code in the front end.

Step (1)

Go to the UI dashboard and click on the view your created earlier called “Renault 5”. Then click on the PEN icon at the top right of the screen (Edit Dashboard). Now move your mouse over to the second column sensor block where you see “Passenger Seat” listed. As you do you will see another pen icon appear. Click on it, and a dialog box will appear.

Step (2)

Now click on where it says “Show Code Editor”

Step (3)

At the top of the code you will see the following lines:

entity: binary_sensor.r5_heated_seat_passenger_rhd_api

name: Passenger Seat

Change these to:

entity: binary_sensor.r5_heated_seat_passenger_lhd_api

name: Driver’s Seat

Step (4)

Save the changes

Step (5)

Repeat the process for the Drivers seat sensor, changing the same two lines to:

entity: binary_sensor.r5_heated_seat_driver_lhd_api

name: Passenger Seat

The code logic above may seem confusing or reversed, but it is simply that Renault treats the seats as simply left and right and does not discern between Driver and Passenger. You will find this is the case in the My Renault App, when setting these options on or off, as it shows the Driver on the left seat and the passenger on the right. Just implement the changes shown above and you will always see the seat options you selected in the app shown correctly in the dashboard

40kWh Battery

If your car is equipped with the 40kWh battery there are a number of changes you need to make. I will go through each in turn

Step (1)

In Home Assistant go to the “Settings” menu, then select “Automations & Scenes”. Then click on automation (01). Now click on three dots at the top right of the screen and select “Edit in YAML”.

Step (2)

Once the code is listed, you will need to look for these two lines:

cap_kwh: "{{ states('input_number.r5_battery_capacity_kwh') | float(52) }}"

avail_energy_s: "{{ states('sensor.r5_available_energy_52_api') }}"

Simply change the number 52 to 40 in each of those lines, save the automation and exit.

Step (3)

Repeat this process for automation (14)

Look for this line:

now_kwh_s: "{{ states('sensor.r5_available_energy_52_kwh_api') }}"

As before change 52 to 40, save the automation and exit

Step (4)

In File Editor, navigate to the “helpers” folder you created earlier. Open the file “input_number.yaml” and look for this entry:

r5_battery_capacity_kwh:

name: R5 Battery Capacity (kWh)

min: 0

max: 52

step: 0.1

unit_of_measurement: kWh

initial: 52

Change ‘max:’ to 40 and ‘initial:’ to 40. Save the file and exit.

Step (5)

In File Editor, navigate to the “scripts” directory you created earlier. Open the file “renault5-scripts.yaml”

Look for the following 7 (seven) separate lines that will appear in this order:

cap: "{{ states('input_number.r5_battery_capacity_kwh') | float(52) }}"

end_energy_s: "{{ states('sensor.r5_available_energy_52_kwh_api') }}"

cap_kwh: "{{ states('input_number.r5_battery_capacity_kwh') | float(52) }}"

end_energy_api_s: "{{ states('sensor.r5_available_energy_52_kwh_api') }}"

cap_kwh: "{{ states('input_number.r5_battery_capacity_kwh') | float(52) }}"

cap: "{{ states('input_number.r5_battery_capacity_kwh') | float(52) }}"

end_energy_api_s: "{{ states('sensor.r5_available_energy_52_kwh_api') }}"

As before change each instance of 52 to 40, save the file and exit

Step (6)

In File Editor go to the “templates” folder you created earlier. Open the file “r5_template_sensors.yaml”.

Look for this sensor “R5 Charging Rate (kW)”. Scroll down the code for this sensor and find the following line:

{% set now_kwh = states('sensor.r5_available_energy_52_kwh_api') | float(-1) %}

As before change 52 to 40

Step (7)

Look for this sensor “R5 Charge Average Power (TMP)”. Scroll down the code for this sensor and find the following 2 (two) lines:

{% set now_kwh = states('sensor.r5_available_energy_52_kwh_api') | float(-1) %}

{% set now_kwh = states('sensor.r5_available_energy_52_kwh_api') | float(-1) %}

As before change 52 to 40 on both lines

Step (8)

Save the file and exit

Step (9)

Go back to the UI and select your Renault 5 front end card view. Click on the PEN icon at the top right of the screen (Edit Dashboard). Move your mouse over the block containing the Vehicle Status sensors. You will see a PEN icon appear. Click on it and a pop up dialog will appear called “Vertical Stack Card Configuration” and the numbers 1 & 2.

Step (10)

Click on “2” and you will be greeted by a host of numbers from 1-12, each representing a sensor. We need to change a couple of them to suit the 40kWh instance.

Step (11)

Click on  number “2” of 12 and you will see a “{}” and the word “Entity”. Click on the “{}”. You are now seeing the code just for that sensor. On the second line (line 2) change the entity to: sensor.r5_available_energy_40_kwh_api

Step (12)

Next scroll down to line 44 and do the same (effectively just overwrite “52” with “40”). Once done click “Save” and you are done.

KM Readout

Step (1)

If you prefer to have km displayed for battery autonomy (range) as opposed to miles, go back to the UI card view for the Renault and click on the Pen icon at the top right of the screen, to enter edit mode.

Step (2)

Move your mouse over the block containing the Vehicle Status sensors. You will see a PEN icon appear. Click on it and a pop up dialog will appear called “Vertical Stack Card Configuration” and the numbers 1 & 2.

Step (3)

Click on “2” and you will be greeted by a host of numbers from 1-12, each representing a sensor. We need to change one sensor to suit the KM reading preference.

Step (4)

Click on number “3” of 12 and you will see a “{}” and the word “Entity”. Click on the “{}”. You are now seeing the code just for that sensor. On the second line (line 2) change the entity to: sensor.r5_battery_autonomy_km_ api

Step (5)

Next scroll down to line 43 and do the same (effectively just overwrite “mi” with “km”). Once done click “Save” and you are done.

As a final step, restart Home Assistant again to update the system and so that it can load the new files properly.

Testing the card.

Once restarted, you can test if the data for your car is being retrieved successfully thus:

Paste the following code into "Developer Tools > Templates" and you should see your car's data for each RAW sensor.

### EV Settings ###

{{ state_attr('sensor.r5_ev_settings_raw','data')}}

### SOC Levels ###

{{ state_attr('sensor.r5_soc_levels_raw','data')}}

### Location ###

{{ state_attr('sensor.r5_location_raw','data')}}

### HVAC ###

{{ state_attr('sensor.r5_hvac_raw','data')}}

### Cockpit ###

{{ state_attr('sensor.r5_cockpit_raw','data')}}

### Charges ###

{{ state_attr('sensor.r5_charges_raw','data')}}

### Battery ###

{{ state_attr('sensor.r5_battery_raw','data')}}

What you are looking for is "OK", "True", "Live" and "No Errors". This indicates that your credentials file is valid, your current token is active, you are logged in to the API and that your data is current and not being fed to the sensors as cached data from a previous API call.

Under normal circumstances the automations will take care of your token by attempting to refresh it or log you back in to the API automatically. If you do see "False", in any returned data endpoints, it means that your token has expired, cached data is being used to feed the sensors or that you are no longer logged in to the Renault API.

In this case, the easiest remedy is to log back in via the CLI in the "Advanced SSH & Web Terminal" Home Assistant add-on. Details on how to do this are as explained earlier in this guide.

Final Points

Much of this guide assumes you have configured the setup for this card correctly and that you have entered the correct API credentials and installed all the various addons such as HACS, card-mod, button cards, mushroom cards and the Google Map card. If these are not correctly configured beforehand, this Renault 5 card simply will not work!

If you spot any errors or bugs, please raise an issue via GitHub. If you have any feature requests, then submit a request and I will consider it, if possible.

Good luck and please be sure to read the user guide document, that accompanies these instructions. I hope you enjoy your new Renault 5 HA card!

Regards

Rod